KFall 数据集是一个公开可用的数据集，专为使用可穿戴传感器数据检测老年人跌倒而设计。它包括从可穿戴设备中的多个传感器收集的数据。创建该数据集是为了支持跌倒检测系统的开发。

数据集：

1.使用的传感器：

- 加速度计（测量线性加速度）

- 陀螺仪（测量角速度）

- 磁力计 （测量方向）

2. 数据收集设置：

根据场景，佩戴在腰部和其他位置。

在受控条件下与人类参与者模拟跌倒和日常活动收集。

采样率：100 Hz（因版本而异）。

3. 记录的活动：

跌倒：向前、向后、向侧面和坐着跌倒。

非跌倒活动：走路、站、坐、躺 等其他姿势之间的转换。

4. 数据格式：

时间序列格式，每行表示给定时间步的传感器读数。

用于监督学习的标记数据，将每个实例分类为跌倒或非跌倒。

5. 用途：

适用于跌倒检测研究中的机器学习和深度学习模型。

用于基于 IoT 的医疗保健解决方案，用于老年人监测。

本项目的主要目标是建立一个机器学习模型，该模型通过传感器数据，判断老人是否发生跌倒

- 理解数据结构，清洗数据（如果需要的话）

- 随机选取一部分数据作为训练集，一部分作为验证集，一部分作为测试集（如果需要的话）

- 设计机器学习分类算法并编程实现

- 撰写完整的报告

# Description of KFall Dataset Files

The KFall dataset contains two main directories:
1️⃣ Label Data (label_data/) → Contains fall event details for each participant.
2️⃣ Sensor Data (sensor_data/) → Contains raw sensor readings from multiple trials.

## 1. Label Data (label_data/)

📁 Path: /KFall Dataset/label_data/
📄 Files: SA06_label.xlsx to SA38_label.xlsx (One for each participant)

🔹 Each file (SAxx_label.xlsx) contains:

File name: SAxx = Unique ID for each participant (e.g., SA06, SA07, … SA38)

| Column | Description |
| --- | --- |
| Task ID | Trial number for fall (e.g. F01, F02, F03, …) The number in ( ) is mapped to task ID of performed in sensor data (e.g. 20 = T20, 21 = T21, 22 =T22, …) |
| Description | Description of fall |
| Trial ID | Trail number under this description, 1 = R01, 2=R02, 3=R03, … |
| Fall_onset_frame | Frame number where pre-fall starts |
| Fall_impact_frame | Frame number where fall impact occurs |

👉 Purpose: These files provide the ground truth labels for each trial.

## 2. Sensor Data (sensor_data/)

📁 Path: /KFall Dataset/sensor_data/
📂 Folders: SA06/ to SA38/ (One for each participant)

🔹 Each folder (SAxx/) contains multiple CSV files (SAxxTyyRzz.csv)

📄 File Naming Format:
SA06T01R01.csv → (SAxx = participant, Tyy = task, Rzz = trial)

| Txx | Task number performed (e.g., T01 for walking, T02 for sitting, etc.) |
| --- | --- |

🔹 Each CSV file contains:

| Column | Description |
| --- | --- |
| Frame Number | The frame index of the sensor reading |
| Timestamp | Time in milliseconds |
| AccX, AccY, AccZ | Accelerometer readings (X, Y, Z axis) |
| GyroX, GyroY, GyroZ | Gyroscope readings (X, Y, Z axis) |
| EulerX EulerY EulerZ | EulerX EulerY EulerZ |